Posttest

Anonymous ID: ____________________

Instructions: This test assesses your understanding of relevant concepts after the PPE instructional activity and does not count toward your course grade. It contains 10 questions worth 1 point each, for a total of 10 points. The test takes approximately 10 min. Please complete it independently.

1. If the theoretical specific capacity of the positive-electrode active material remains unchanged, which change can directly increase the theoretical capacity of the positive electrode?

A. Decrease the discharge current

B. Increase the ionic conductivity of the electrolyte

C. Decrease the particle radius (with mass unchanged)

D. Increase the mass of positive-electrode active material

2. All other conditions being equal, which change is most likely to worsen the high-rate discharge performance of an electrode?

A. Increase the effective transport capability of the electrolyte

B. Increase the tortuosity of electrolyte transport pathways

C. Decrease the electrode thickness

D. Increase the solid-state lithium diffusion coefficient

3. For the same material and solid-state diffusion coefficient, what is the most direct effect of decreasing the active-particle radius?

A. Shorten the solid-state diffusion distance within the particles

B. Reduce the thickness of the entire electrode

C. Increase the intrinsic ionic conductivity of the electrolyte

D. Change the equilibrium potential of the active material

4. For the same cell at the same SOC and temperature, if polarization decreases during discharge, the terminal voltage will generally:

A. Reach the cutoff voltage sooner

B. Necessarily drop to zero

C. Move closer to the equilibrium voltage

D. Be lower than when polarization is greater

5. At the same discharge current, one set of simulations has a lower effective ionic conductivity of the electrolyte and a more pronounced terminal-voltage drop. Which explanation is most reasonable?

A. An increased potential drop in the electrolyte or greater transport-related polarization

B. An automatic decrease in the theoretical specific capacity of the active material

C. An automatic increase in the active-particle radius with current

D. An automatic increase in electrode thickness during discharge

6. Two electrodes have different pore structures, but their terminal-voltage curves are similar under a given discharge condition. Which conclusion is most reasonable?

A. Their local ion concentration distributions must be identical

B. Their internal reaction distributions must be identical

C. Their structural differences have no effect under any operating conditions

D. Their macroscopic terminal-voltage responses are similar under the present conditions, but their local states may still differ

7. With the positive-electrode area and the active-material volume fraction unchanged, the electrode thickness is increased. Which statement is most reasonable?

A. The theoretical areal capacity of the positive electrode must decrease

B. The ion-transport distance in the electrolyte must decrease

C. The theoretical areal capacity of the positive electrode increases, but high-rate performance does not necessarily improve

D. The equilibrium potential of the active material must increase

8. The same cell is discharged from the same initial SOC, at the same temperature and cutoff voltage. More capacity is delivered at a lower discharge rate. Which explanation is most reasonable?

A. A lower rate automatically increases the mass of active material

B. Lower polarization at the lower rate allows more capacity to be delivered before the cutoff voltage is reached

C. A lower rate changes the theoretical specific capacity of the active material

D. A lower rate automatically changes the specified cutoff voltage

9. When both the type and mass of active material remain unchanged, lowering only the discharge rate will increase the theoretical capacity of the positive electrode.

○ True        ○ False

10. All other conditions being equal, increasing the active-particle radius may make solid-state diffusion limitations within the particles more pronounced.

○ True        ○ False

Answer Key (For Instructors)

| Question | Answer |
| --- | --- |
| 1 | D |
| 2 | B |
| 3 | A |
| 4 | C |
| 5 | A |
| 6 | D |
| 7 | C |
| 8 | B |
| 9 | False |
| 10 | True |