Pretest

Anonymous ID: ____________________

Instructions: This test assesses your understanding of relevant concepts before the PPE instructional activity and does not count toward your course grade. It contains 10 questions worth 1 point each, for a total of 10 points. The test takes approximately 10 min. Please complete it independently.

1. With the type of active material and its theoretical specific capacity held constant, which change would directly increase the theoretical capacity of the positive electrode?

A. Increase the discharge current

B. Increase the mass of positive-electrode active material

C. Increase the active-particle radius (with mass unchanged)

D. Shorten the test duration

2. All other conditions being equal, which change is most likely to worsen the high-rate discharge performance of an electrode?

A. Increase the effective ionic conductivity of the electrolyte

B. Decrease the active-particle radius

C. Increase the solid-state lithium diffusion coefficient

D. Decrease the effective ionic conductivity of the electrolyte

3. With the solid-state diffusion coefficient unchanged, increasing the active-particle radius generally causes the characteristic diffusion distance of lithium ions within the particles to:

A. Decrease

B. Remain unchanged

C. Increase

D. Become equal to the electrode thickness

4. For the same cell at the same SOC and temperature, if the discharge current increases and polarization becomes more pronounced, the terminal voltage will generally:

A. Decrease

B. Increase

C. Remain unchanged

D. Automatically equal the equilibrium voltage

5. Two positive electrodes have the same active-material mass and discharge current. One has lower porosity and more restricted ion transport. This electrode is more likely to exhibit:

A. Reduced electrolyte transport polarization

B. A change in the equilibrium potential of the active material

C. Increased electrolyte-transport-related polarization

D. An inevitable increase in the solid-state diffusion coefficient

6. The simulated discharge-voltage curves for two different particle sizes are very similar. Which conclusion is most justified?

A. Their internal lithium concentration distributions must be identical

B. Their macroscopic terminal-voltage responses are similar under the present operating conditions

C. Their solid-state diffusion processes are exactly the same

D. Particle size has no effect under any operating conditions

7. The electrode area and thickness, as well as the specific capacity of the active material, remain unchanged. If the mass of positive-electrode active material increases while pore space decreases, which statement is most reasonable?

A. Both theoretical capacity and rate performance must improve

B. The theoretical capacity of the positive electrode must decrease

C. Lower porosity does not affect ion transport

D. The theoretical capacity of the positive electrode increases, but high-rate performance does not necessarily improve

8. The same cell is discharged from the same initial SOC, at the same temperature and cutoff voltage. At a higher discharge rate, the cutoff voltage is reached after less capacity has been delivered. Which explanation is most reasonable?

A. Greater polarization at the higher rate causes the terminal voltage to reach the cutoff after less capacity has been delivered

B. A higher rate necessarily increases the particle radius

C. A higher rate changes the type of active material

D. A higher rate automatically lowers the specified cutoff voltage

9. When both the type and mass of active material remain unchanged, increasing only the average particle radius will increase the theoretical capacity of the positive electrode.

○ True        ○ False

10. All other conditions being equal, a decrease in the effective ionic conductivity of the electrolyte may intensify transport-related polarization during discharge.

○ True        ○ False

Answer Key (For Instructors)

| Question | Answer |
| --- | --- |
| 1 | B |
| 2 | D |
| 3 | C |
| 4 | A |
| 5 | C |
| 6 | B |
| 7 | D |
| 8 | A |
| 9 | False |
| 10 | True |